Name:  Samrathpreet Singh Randhawa

Roll No.: 1801CS43

CS-226 (Switching Theory Lab)

Lab-8

Question 0) Basic sequential elements J-K, Q and D flip flops.

JK Flip Flop

|  | Clock |  | J |  | K | Q(t) |  | Q’(t) | State |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  | X |  | X | Q(t-1) |  | Q’(t-1) | Memory |  |
|  | 1 | 0 |  | 0 |  | Q(t-1) |  | Q’(t-1) | Memory |  |
|  | 1 | 0 |  | 1 |  | 0 | 1 |  | Reset |  |
|  | 1 | 1 |  | 0 |  | 1 | 0 |  | Set |  |
|  | 1 |  | 1 |  | 1 | Q’(t-1) |  | Q(t-1) | Toggle |  |

Truth Table

Test Case:

- Set J = 1, K = 1 followed by Clock = 1, notice Value stored = 1 (Set State)

- Set Clock = 0, change J and K randomly, no change in Value stored (Memory State)

- Set J = 0, K = 1, then Clock = 1, notice Value stored = 0 (Reset State)

- Set Clock = 0

- Set J = 1, K = 1, and keep toggling the clock, notice value stored also toggles (Toggle State)

SR Flip Flop

|  | Clock |  | S |  | R | Q(t) |  | Q’(t) | State |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  | X |  | X | Q(t-1) |  | Q’(t-1) | Memory |  |
|  | 1 | 0 |  | 0 |  |  |  | Forbidden |  |  |
|  | 1 | 0 |  | 1 |  | 0 | 1 |  | Reset |  |
|  | 1 | 1 |  | 0 |  | 1 | 0 |  | Set |  |
|  | 1 |  | 1 |  | 1 | Q(t-1) |  | Q’(t-1) | Memory |  |

Truth Table

Test Case:

- Set S = 1, followed by Clock = 1, notice Value stored = 1 (Set State)

- Set Clock = 0, change S and R randomly, no change in Value stored (Memory State)

- Set S = 0, R = 1, then Clock = 1, notice Value stored = 0 (Reset State)

- Set Clock = 0

- Set S = 1, R = 1, then Clock = 1, no change in Value stored (Memory State)

D Flip Flop

|  | Clock |  | D |  | Q(t) | State |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  | X |  | Q(t-1) | Memory |  |
|  | 1 | 0 |  | 0 |  | Reset |  |
|  | 1 |  | 1 |  | 1 | Set |  |

Truth Table

Test Case:

- Set Clock = 1, D = 1, notice Value Stored = 1

- Set Clock = 0, Value stored doesn’t change on changing D (Memory)

- Set D = 1, Clock = 1, notice Value Stored = 1

T Flip Flop

|  | Clock |  | T |  | Q(t) | State |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  | X |  | Q(t-1) | Memory |  |
|  | 1 | 0 |  |  | Q(t-1) | Memory |  |
|  | 1 |  | 1 |  | Q’(t-1) | Toggle |  |

Truth Table

Test Case:

- Set Clock = 1, T = 0, notice no change in Value Stored

- Set Clock = 0, Value stored doesn’t change on changing T (Memory)

- Set T = 1, Clock = 1, and keep toggling the clock, notice Value stored also toggles

Question 1) 8 Bit Shift Register using flipflop is implemented in file p1. circ

Question 2) Simulated 4-bit Parallel Input Parallel Output (PIPO) register and submitted in p2. circ

Question 3) Submitted in p3. circ

4 Bit counter using T-Flip Flops

6 Bit counter using T-Flip Flops

Question 4) Simulated 8-bit Multifunction Shift register using Logisim submitted in p4. circ

4bit multi-function register

8bit version multi-function register

Question 5) Implemented a 16 x 16 register file submitted in p5. circ